Lipid analysis protocol Degerhamn 2014-11-28

Fredrik Svensson.

Based on the Folch method (Folch et al, 1957).

Chemicals;

Prepare a 2:1 Chloroform : Methanol solution

Chloroform

0.73 % NaCl solution

Materials;

Freeze dried algae biomass of known quantity.

Falcon tubes

Aluminium foil

Aluminium cups

Glass tubes that can be centrifuged

20 ml syringe

25 mm w/0.2 PTFE syringe filters

When working with chloroform and organic solvents, work in a fume hood. Inform other people in the lab about what chemicals you are using. Wear gloves, lab coat and safety goggles.

- Suspend the pellet in 4.5 ml Chloroform:Methanol 2:1.

- Vortex the samples.

- From this point, protect the samples from exposure to light by wrapping them in tinfoil.

- Sonicate the samples 2 min 50 % capacity using Vibracell VCX 130. Keep samples on ice.

- Store samples cold and dark 8 °C until next day.

- Centrifuge the samples. Discard the upper aqueous supernatant (if any) gently without disturbing the lower Chloroform/MeOH supernatant. Collect the Chloroform/ MeOH supernatant in new clean tubes that can be centrifuged.

- Repeat the extraction process a total 3 times.

- To the recovered supernatant, add 3.375 ml 0,73% NaCl solution.

- Final proportions shall be Chloroform:MeOH:H2O 8:4:3.

- Vortex until homogenous.

- Centrifuge 10 min 4000 rpm. Two separate phases are formed; one aqueous and one lipid phase.

- Remove and discard the aqueous upper phase without interfering with the lower lipid phase.

- Collect the lipid supernatant and filter through a 25 mm syringe filter 0.2 μm PTFE membrane into pre-weighed aluminium cups. Wash the tube and syringe with chloroform.

- Add a few drops of MeOH to evaporate traces of water.

- Evaporate the samples in a fume hood at room temperature.

- When samples appear dry and the chloroform have evaporated, dry the samples in an oven at 60 °C until constant weight.

References;

Folch J, Lees M, Stanley GHS (1957) A simple method for the isolation and purification of total lipides from animal tissues. J Biol Chem 226:497–509

Bligh EG, Dyer WJ (1959) A rapid method of total lipid extraction and purification. Can J Biochem Physiol 37:911–917

Ryckebosch E, Muylaert K, Foubert I. (2011) Optimization of an analytical procedure for extraction of lipids from microalgae. J Am Oil Chem Soc DOI 10.1007/s11746-011-1903-z

Iverson SJ, Shelley LC, Cooper MH. (2011) Comparison of the Bligh and Dyer and Folch methods for total lipid determination in a broad range of marine tissue. Lipids vol. 36 no. 11.

Axelsson M, Gentili F. (2014) A single-step method for rapid extraction of total lipids from green microalgae. PLoS ONE 9(2): e89643. doi:10.1371/ journal.pone.0089643