Protein analysis protocol Degerhamn 2014-11-28

Fredrik Svensson.

Based on Lowry et al 1951, and using a ready-made protein bio assay.

Chemicals;

0.5 M NaOH

Bio-Rad DC Protein Assay

- Bovine serum albumin standard

- Reagent A, alkaline copper tartrate solution

- Reagent B, Folin´s Ciocalteu reagent dilution

Materials;

Freeze dried algae biomass of known quantity.

Glass tubes that can be centrifuged

Aluminium foil

Pipette 100 μl

Pipette 4.0 ml

Water bath set at 90 °C

Spectrophotometer

Folin´s Ciocalteu reagent contains Phenol. When working with Phenol, work in a fume hood. Inform other people in the lab about what chemicals you are using. Wear neoprene gloves, lab coat and safety goggles. Pipet tips, gloves and other contaminated debris should be collected as hazardous waste.

- Re-suspend 5 mg biomass samples in 2 ml 0.5 M NaOH.

- Seal the glass tubes with aluminium foil and parafilm paper

- Vortex samples gently.

- Water bath 90 °C 1 h.

- Let samples cool to room temperature.

- Prepare a standard curve using the supplied Bovine serum standard. Prepare 5 dilutions ranging from 0.2 to 1.0 mg/ml protein. Prepare a blank.

- Centrifuge the samples 10 min at 4000 rmp.

- Transfer 100 μl of the supernatant into clean glass tubes.

- Add 500 μl Reagent A to each sample.

- Vortex samples gently.

- Add 4.0 ml Reagent B and vortex immediately.

- Wait for ≈15 min.

- Measure absorbance in a spectrophotometer at 750 nm.

- Compare your samples to the standard curve.